MultiCode Keyless Entry Programming Instructions

Before you begin you must first determine your ‘Radio Code’ (you will need it for Step 1-C in the programming phase.) To do this, remove the battery cover on your hand-held transmitter or look at the dip-switches on the receiver board located on the back of the garage door opener and write down the numbers corresponding to the dip-switches that are in the ‘ON’ position .  (The ‘ON’ position being when the switches are pressed down towards the numbers) If switch 10 is ‘ON’, use 0 as the corresponding number.  Example: 2,5,7,10 are ‘ON’ – your ‘Radio Code’ would be 2,5,7,0.

Read through the entire programming instructions before attempting to program your keyless entry, as you will only have 25 seconds to complete each step.  Failing to do so in the allotted time will cause your keyless entry to ((lock up)) for 5 minutes.

STEP 1:  Radio Code Programming

A:  Press ‘Radio Learn’ twice

B:  Press 1-2-3-4 (factory code)

C:  Enter your ‘Radio Code’

D:  Press ‘Radio Learn’ to end programming

**If the indicator LED blinks rapidly for 4 seconds, there was an error in programming.  Remove the battery, press any button for 5 seconds then wait for 30 seconds before re-installing the battery and begin the programming sequence again.

STEP 2:  Access Code Programming

A:  Choose any 4-digit PIN number (write it down for future reference.)

B:  Press ‘Access Learn’ twice

C:  Press 1-2-3-4 (factory setting)

D:  Enter your new 4-digit PIN number

E:  Press ‘Access Learn’ to end programming