# Secure Code Keyless Entry programming/training (NEW MODELS)

To clear all memory and to return to the factory setting (Rolling Code models):

Press and hold both the ‘Radio Learn’ and ‘Access Learn’ buttons for 10 seconds or until the lights on the keypad go off.

To train to a Rolling Code Garage Door Opener (NEW MODELS):

- Make sure the Keyless Entry is in the factory setting. If necessary clear the memory as described above.

- Press the ‘Access Learn’ button twice on the keypad.

- Press the numbers 1, 2, 3, 4.

- Press the numbers of your new access code.

- Press the Access learn button once.

- Press the REMOTE ‘R’ button on the back of the Garage Door Opener.

- While the lights on the front of the garage door opener are still lit, on the keyless entry press in your new four digit access code.

- If everything to this point was done correctly the lights on the front of the garage door opener should blink on and off at. If they do then the opener accepted the keypad code. If they do not blink at then either you waited too long to enter the access code on the keyless entry, or you performed one of the steps incorrectly.

- Wait for the lights on the keypad to go out, and then test the keypad by entering your access code. The garage door opener should activate.